UDS Discovery Phase (Caring Caribou + iso14229 Linux Server on vcan0)

Step-by-step guide for your current Kali VM setup

1. Scope

This document captures the exact, working “UDS discovery” workflow you reached in your Kali VirtualBox lab:
- iso14229 Linux example runs as the ECU (server) on vcan0
- Caring Caribou acts as the tester (client) and performs UDS discovery to identify responders and addressing
- SocketCAN (vcan0) is used, with python-can configured via .canrc

2. Prerequisites (one-time setup)

You should already have these installed, but this is the clean checklist:

Kali Linux VM (VirtualBox) with sudo access

can-utils (candump, cansend, isotpsend)

Caring Caribou v0.7 installed and working

iso14229 repo cloned and linux_server example built (binary name: server)

Kernel modules available: vcan and can-isotp

Install/verify required packages:

Terminal 1:
sudo apt update
sudo apt install -y can-utils git make gcc

3. Terminal layout (recommended)

| Terminal | Purpose |
| --- | --- |
| Terminal 1 | System setup + interface/config checks |
| Terminal 2 | Monitor traffic (candump vcan0) |
| Terminal 3 | Run iso14229 ECU server (sudo ./server vcan0) |
| Terminal 4 | Run Caring Caribou UDS discovery commands |

4. Bring up vcan0 and ISO-TP (every reboot)

Terminal 1:
sudo modprobe vcan
sudo modprobe can-isotp

sudo ip link add dev vcan0 type vcan 2>/dev/null || true
sudo ip link set up vcan0

ip link show vcan0

Expected: vcan0 appears as UP. If it is DOWN, rerun “ip link set up vcan0”.

5. Configure python-can for Caring Caribou (critical)

Caring Caribou uses python-can. If python-can does not know the interface/channel, you get:
CanInterfaceNotImplementedError: Unknown interface type "None".

Fix: create a .canrc for BOTH your user and root (because you run with sudo).

Terminal 1:
# For your user
printf "[default]\ninterface = socketcan\nchannel = vcan0\n" > ~/.canrc
cat ~/.canrc

# For sudo/root (recommended)
sudo bash -lc 'printf "[default]\ninterface = socketcan\nchannel = vcan0\n" > /root/.canrc'
sudo bash -lc 'cat /root/.canrc'

Expected .canrc contents:

[default]
interface = socketcan
channel = vcan0

6. Start traffic monitor

Terminal 2:
candump vcan0

Leave Terminal 2 running during discovery to confirm requests and responses are present.

7. Start the iso14229 ECU server

Your iso14229 linux_server example initializes ISO-TP with:
- Request ID (Tester -> ECU): 0x7E0
- Response ID (ECU -> Tester): 0x7E8
- Functional address: 0x7DF

Terminal 3:
cd ~/iso14229-lab/iso14229/examples/linux_server
sudo ./server vcan0

If it starts successfully, keep it running while you do discovery.

8. UDS Discovery phase (Caring Caribou)

Goal: identify which arbitration IDs respond to UDS requests, so you can choose the correct src/dst pair for later service probing.
In this lab, the server uses 0x7E0/0x7E8, but you should still run discovery as the formal first step.

Terminal 4:
# Basic discovery
sudo caringcaribou uds discovery

# If you want to ignore known IDs (optional)
# sudo caringcaribou uds discovery -blacklist 0x123 0x456

# If your lab becomes noisy, auto-blacklist IDs that spam responses (optional)
# sudo caringcaribou uds discovery -autoblacklist 10

What to watch for during discovery:

Terminal 4 output listing responding IDs (responders)

Terminal 2 candump showing traffic on 0x7E0 (requests) and 0x7E8 (responses)

If you see only requests (no responses), the server is not running or IDs/channel are wrong

9. Record discovery results (what to write down)

When discovery succeeds, record these items (this becomes your baseline for the rest of fuzzing):

CAN interface: vcan0

UDS physical request ID (tester -> ECU): 0x7E0

UDS physical response ID (ECU -> tester): 0x7E8

UDS functional address (broadcast): 0x7DF (optional/for later tests)

Caring Caribou version: v0.7

python-can config: interface=socketcan, channel=vcan0 (user + root)

10. Troubleshooting checklist (fast)

A) Error: CanInterfaceNotImplementedError: Unknown interface type "None"

Fix: ensure ~/.canrc and /root/.canrc exist and both specify socketcan + vcan0. Then rerun discovery.

B) Discovery finds nothing (no responders)

Confirm vcan0 is UP: ip link show vcan0

Confirm server is running in Terminal 3

Confirm you see traffic in Terminal 2 candump

Confirm can-isotp is loaded: lsmod | grep isotp (if empty: sudo modprobe can-isotp)

C) Discovery sends traffic but server does not respond

Fix: verify server IDs in main.c match the discovery/test pair. For your server example: 0x7E0/0x7E8.

D) Sudo vs user environment mismatch

If commands work without sudo but fail with sudo, copy .canrc to /root/.canrc and run again.

11. Next steps (after discovery)

Once discovery confirms the responder IDs, you can proceed to service enumeration and targeted fuzzing modules.
Examples (use the IDs you discovered):
- services: sudo caringcaribou uds services 0x7E0 0x7E8
- subservices: sudo caringcaribou uds subservices 0x00 0x10 0x7E0 0x7E8
- dump DIDs: sudo caringcaribou uds dump_dids 0x7E0 0x7E8