UDS Discovery & Service Identification (uds-server + Caring Caribou)

Environment: Kali Linux (Oracle VirtualBox) + SocketCAN vcan0 + Caring Caribou v0.7 + zombieCraig/uds-server

# Goal

Discover a UDS diagnostics server (ECU simulator) running on a virtual CAN interface (vcan0), then identify which UDS services it supports using Caring Caribou.

# Prerequisites

Install required packages (run once):

sudo apt update
sudo apt install -y git build-essential can-utils

Confirm Caring Caribou is installed and accessible:

caringcaribou --help

# Step-by-step (Terminal by Terminal)

## Terminal 1 — Create and bring up vcan0

Create the virtual CAN interface used inside the VM:

sudo modprobe vcan
sudo ip link add dev vcan0 type vcan
sudo ip link set up vcan0
ip -details link show vcan0

Expected: vcan0 shows as UP.

## Terminal 2 — Clone and build uds-server

Clone the repository and compile:

git clone https://github.com/zombieCraig/uds-server.git
cd uds-server
make

Expected: compilation completes without errors and produces the uds-server binary.

## Terminal 3 — Start the UDS ECU simulator on vcan0

Run the simulator and keep this terminal open (it must stay running):

./uds-server vcan0

Expected: the simulator starts and listens on vcan0.

## Terminal 4 — Optional: Sniff CAN traffic (sanity check)

Monitor CAN frames to confirm requests and responses:

candump vcan0

Leave candump running while you test in other terminals.

## Terminal 5 — UDS Discovery (find client/server IDs)

Run UDS discovery with Caring Caribou:

caringcaribou uds discovery

Example of a successful discovery result (your run):

Identified diagnostics:

+------------+------------+
| CLIENT ID  | SERVER ID  |
+------------+------------+
| 0x00000710 | 0x0000077a |
| 0x000007df | 0x0000077a |
| 0x000007e0 | 0x0000077a |
+------------+------------+

Interpretation: the diagnostics server responds from 0x77A. It accepts requests sent to 0x710, 0x7DF (functional), and 0x7E0.

## Terminal 5 — Service scan (identify supported UDS services)

Choose one request/response pair discovered above. A clean physical pair from your discovery is:
Request (tester→ECU): 0x710
Response (ECU→tester): 0x77A

Scan for supported services:

caringcaribou uds services 0x710 0x77A

Example result (your run):

Supported service 0x10: DIAGNOSTIC_SESSION_CONTROL

Interpretation: the simulator supports UDS service 0x10 (Diagnostic Session Control). Other services were not reported as supported.

# Notes & Troubleshooting

If discovery finds nothing:

• Confirm uds-server is still running on vcan0 (Terminal 3).

• Confirm vcan0 is UP (Terminal 1).

• Use candump (Terminal 4) to verify traffic is present.

If you want to validate the link manually (optional):

# In one terminal:
candump vcan0

# In another terminal (send a basic session control request to 0x710):
cansend vcan0 710#021001