Caring Caribou (Tester)

↓

SocketCAN (vcan0)

↓

ISO-TP Layer

↓

uds-to-go (ECU Simulator)

0) Install Requirements (ONE TIME)

sudo apt update

sudo apt install -y can-utils cmake g++ build-essential python3-pip

pip install caringcaribou

1) Create Virtual CAN Interface

sudo modprobe vcan

sudo ip link add dev vcan0 type vcan 2>/dev/null || true

sudo ip link set vcan0 up

Verify:

ip link show vcan0

2) Build uds-to-go (ONE TIME)

cd ~

git clone https://github.com/astand/uds-to-go.git

cd uds-to-go

cmake -S src -B build

cmake --build build

3) Start ECU Simulator (Terminal A)

Open Terminal A:

cd ~/uds-to-go

./build/ecu-server-test -iface vcan0 -phys 0x710 -resp 0x711 -blksize 25 -stmin 0

You should see:

PHYS = 710

RESP = 711

Started successfully

1 : Session Control

2 : Read DID

3 : Tester Present

4 : Routine request

5 : Security control

4) Enable UDS Services on the ECU

In Terminal A, type these one by one and press Enter:

1

1

2

3

4

5

This enables:

| Service | Hex | Meaning |
| --- | --- | --- |
| Session Control | 0x10 | Change diagnostic session |
| Read DID | 0x22 | Read ECU data |
| Tester Present | 0x3E | Keep session alive |
| Routine Control | 0x31 | Run ECU routines |
| Security Access | 0x27 | Seed/Key security |

Leave Terminal A running.

5) Sniff Traffic (Optional but Recommended)

Open Terminal B:

candump -L vcan0

6) Test ECU Connectivity (Terminal C)

Open Terminal C:

caringcaribou uds testerpresent -dur 5 0x710

7) Enumerate Supported UDS Services (MAIN GOAL)

Still in Terminal C:

caringcaribou uds services 0x710 0x711

8) Expected Output

You should see something like:

[+] Service 0x10: DiagnosticSessionControl

[+] Service 0x22: ReadDataByIdentifier

[+] Service 0x27: SecurityAccess

[+] Service 0x3E: TesterPresent

[+] Service 0x31: RoutineControl

9) Deep Scan Individual Services

Session Subservices:

caringcaribou uds subservices 0x10 0x00 0x710 0x711

Security Levels:

caringcaribou uds subservices 0x27 0x00 0x710 0x711

Dump DIDs:

caringcaribou uds dump_dids 0x710 0x711