UDS Discovery & Service Identification (UDSim + Caring Caribou)

Environment: Kali Linux (Oracle VirtualBox) + SocketCAN vcan0 + Caring Caribou v0.7 + zombieCraig/UDSim

# Goal

Run UDSim as a simulated ECU on a virtual CAN interface (vcan0), then discover the diagnostics server and identify supported UDS services using Caring Caribou.

# Prerequisites (run once)

1) Create vcan0 (Virtual CAN)

sudo modprobe vcan
sudo ip link add dev vcan0 type vcan
sudo ip link set up vcan0
ip -details link show vcan0

2) Install dependencies and build UDSim

sudo apt update
sudo apt install -y git build-essential can-utils \
  libsdl2-dev libsdl2-image-dev libsdl2-ttf-dev

git clone https://github.com/zombieCraig/UDSim.git
cd UDSim/src
make

Note: If 'make: Nothing to be done for all' appears, UDSim is already built.

# Step-by-step (Terminal by Terminal)

## Terminal 1 — Create the UDSim configuration file

Create a minimal config that simulates a responder on 0x7E8 to requests sent to 0x7E0.

nano ~/udsim.cfg

Paste the following content into the editor:

[7e0]
responder = 0
positiveID = 7e8
negativeID = 7e8
{Packets}
7e0#0210030000000000

[7e8]
responder = 1
{Packets}
7e8#0650030096177000

Save and exit: Ctrl+O → Enter → Ctrl+X

Verify the file exists:

ls -l ~/udsim.cfg

## Terminal 2 — Start UDSim on vcan0 (keep it running)

Launch UDSim using the config file and the vcan0 interface:

cd ~/UDSim/src
./udsim -c ~/udsim.cfg vcan0

Important: UDSim should remain running during discovery and service scanning. If a GUI window appears, keep it open.

## Terminal 3 — Optional: Sniff CAN traffic (sanity check)

Monitor CAN frames to confirm requests/responses:

candump vcan0

Leave this running while testing in other terminals.

## Terminal 4 — Kick-test UDSim (prove it replies)

Send a Diagnostic Session Control request (SID 0x10, subfunction 0x03):

cansend vcan0 7E0#021003

Expected: you see a response from 0x7E8 in candump (Terminal 3).

## Terminal 4 — Run UDS discovery with Caring Caribou

Run the discovery scan:

caringcaribou uds discovery

If discovery fails but you confirmed UDSim responds to cansend, use the known IDs from the config (0x7E0 request, 0x7E8 response) to continue directly with service identification.

## Terminal 4 — Identify supported services (service scan)

Scan supported UDS services using the request/response pair from the UDSim config:

caringcaribou uds services 0x7E0 0x7E8

Expected: Supported service 0x10 (DIAGNOSTIC_SESSION_CONTROL).

# Troubleshooting

1) If UDSim does not start or exits immediately:

• Run UDSim from a desktop session inside VirtualBox (UDSim uses SDL).

• Check for SDL/display errors printed in Terminal 2.

2) If Caring Caribou discovery reports no ECU but cansend works:

• Skip discovery and run services directly on 0x7E0/0x7E8 (as above).

• Optionally inspect discovery options:

caringcaribou uds discovery --help

• Ensure Caring Caribou is using vcan0 (matches your lab .canrc):

cat ~/.canrc 2>/dev/null
cat /root/.canrc 2>/dev/null

3) Quick end-to-end validation (optional):

# Terminal A
candump vcan0

# Terminal B
cansend vcan0 7E0#021003

# Terminal C
caringcaribou uds services 0x7E0 0x7E8