# ✅ FULL REPRODUCIBLE SETUP

Gallia virtual UDS ECU + Caring Caribou testing

## 🧩 1. System prerequisites (once)

sudo apt update
sudo apt install -y \
  python3 python3-venv python3-pip \
  git can-utils

## 🚌 2. Create virtual CAN interface (vcan0)

sudo modprobe vcan
sudo ip link add dev vcan0 type vcan 2>/dev/null || true
sudo ip link set up vcan0
ip link show vcan0

✔ state UNKNOWN is normal

## 🧪 3. Install & run Gallia (Terminal 1)

cd ~
git clone https://github.com/Fraunhofer-AISEC/gallia.git
cd gallia

python3 -m venv venv
source venv/bin/activate
pip install --upgrade pip setuptools wheel
pip install -e .

### ▶ Start Gallia virtual ECU (RNG mode)

gallia script vecu rng "isotp://vcan0?rx_id=1792&tx_id=1800"

📌 Do not close this terminal
 ✔ ECU is now running
 ✔ 0x700 = request ID
 ✔ 0x708 = response ID

## 🔍 (Optional) Verify ECU manually

candump vcan0

cansend vcan0 700#02103E00

You should see a response on 0x708.

## 🧭 4. Install Caring Caribou (Terminal 2)

cd ~
git clone https://github.com/CaringCaribou/caringcaribou.git
cd caringcaribou

python3 -m venv venv
source venv/bin/activate
pip install -r requirements.txt
pip install -e .

## ⚙️ 5. Configure python-can (CRITICAL)

export CAN_INTERFACE=socketcan
export CAN_CHANNEL=vcan0

(Optional – make it permanent)

echo 'export CAN_INTERFACE=socketcan' >> ~/.bashrc
echo 'export CAN_CHANNEL=vcan0' >> ~/.bashrc

## 🚦 6. Test ECU with Caring Caribou

### ▶ Tester Present (keep ECU alive)

caringcaribou uds testerpresent 0x700

### ▶ Discover UDS services

caringcaribou uds services 0x700 0x708

✔ Services appear
 ✔ ECU responds correctly

### ▶ Discover subservices (example)

caringcaribou uds subservices 0x700 0x708 0x10